Car Setup Document

Connecting to the Car

- Open vscode and click “open a remote window” in bottom left corner

- Locate the IP address of the pi via admin portal of the internet router

  - If you have a VPN, use that IP address if possible (Tailscale). I can connect to the pi from anywhere so long as it is connected to Wi-Fi. *preferred method*

  - Backup - Create a personal hotspot on my laptop to create my own VPN for the pi to connect to. This will be slower.

- To run Donkey Car Web:

  - Open the ~/mycar folder in a new terminal

  - Run python manage.py drive

  - Use the IP address of the py and open http://<IP>:8887 (this IP will vary between cars)

  - You can now control the Donkey Car through the web app and view the live camera feed

- To diagnose issues with the display

  - Display not working on PiRacer Pro · Issue #1 · waveshare/pi-display · GitHub

- To diagnose i2c issues

  - Ensure i2c-tools are installed on the PI (sudo apt-get install i2c-tools)

  - Run i2cdetect -y 1 to run the detector on bus 1

  - You should see data in the 0x40, 0x0 element in the table

Getting the Sim Setup

- Cannot run on my laptop. Can connect to my home CPU via Tailscale and run that way

- Otherwise, follow all the instructions on the donkey website